How to upgrade RK3288 Android firmware ？

1.Unzip the file and get the file  . Install the software under this folder  ,click program  DriverInstall  and get this pop-up

Click Install Driver

- Download Android system in IMG format

Start the AndriodTool you have installed.

This is the initial interface

Click Upgrade Firmware,you will have this interface

Click Firmware, select the Android image you just downloaded

3.Connect USB burning cable to the computer with AndriodTool installed. Long press the upgrade button and connect the other side of the USB burning cable to the indicated USB port (beside the VGA) on the RK3288 board at the same time, and then connect power supply to the RK3288 board, release the upgrade button until the AndroidTool recognize the device.

Click the Upgrade to start the upgrade

- 4.Remove the USB burning cable after Upgrade complete